MSRS Report Format Documentation

Emergency Load Response Make-Whole Credits

Version 4

Revision History

| Date | Revision | Description |
| --- | --- | --- |
| 06/25/2010 | 1 | Initial Distribution |
| 08/19/2011 | 2 | Updated XML Column Name for Date to match the format currently used in production. |
| 02/20/2014 | 3 | Updated Description |
| 2/21/2020 | 4 | Removed all references to eSchedule(s) and/or eSchedule(s) reports. |

# Report

MSRS Report Name: Emergency Load response Make-Whole Credits

Report short name for User Interface: Emergency Load Response Make-Whole Credits

Download File Name Abbreviation: EmLRMW

Data Granularity: daily

Frequency: Updated daily

Range Displayed on Report: Start Month through End Month

# Supported Billing Line Items

- Emergency Load Response Credit (2245)

# Report Content Summary

This report displays a customer account’s daily emergency load response make-whole credits payments by registration prior to 3/1/2014.  After 2/1/2014, members will download the Emergency Load Response Credit report to view both the energy and make-whole credits from emergency load response activity.  The credit (2245.02) on this report should be added to the Emergency Load Response Credit (2245.01) which can be found on the Load Response Summary report.  This report will only show credits where the Emergency Load Response Make-Whole Credits <> 0.

# Summary of Changes and Special Logic

- The date range total row will only appear in the online version of the report.  It will not appear in the CSV and XML versions of the report.

- If any of the following columns are not applicable to the resource listed in a given row, 0s will be displayed in these columns: Emergency Bid, RT Shutdown Cost, RT Shutdown Cost Multiplier, Daily Load Response Emergency Credits

- The data on this report is preliminary until the end of the billing cycle.  Final results are based on month-end calculations that could significantly change the preliminary results that are displayed throughout the month.

# Report Columns

The following columns will appear in the body of the report:

| Online and CSV Column Name | XML Column Name | Column Number | Data Type |
| --- | --- | --- | --- |
| Customer ID | CUSTOMER_ID | 4000.01 | INTEGER |
| Customer Code | CUSTOMER_CODE | 4000.02 | VARCHAR2(6) |
| Billing Month | BILLING_MONTH | 4000.03 | DATE (Month, YYYY (ex. January, 2007) in online and CSV formats, YYYY-MM in XML format) |
| Date | DATE | 4000.04 | VARCHAR2(40) mm/dd/yyyy HH24 format (Displays first hour of the day in GMT Format) |
| Registration ID | REGISTRATION_ID | 4000.93 | NUMBER |
| End Use Customer | END_USE_CUSTOMER | 4000.95 | VARCHAR2(40) |
| Emergency Bid ($) | LR_EMERGENCY_BID | 2245.11 | NUMBER(22,2) |
| RT Shutdown Cost | RT_SHUTDOWN_COST | 2376.11 | NUMBER(22,2) |
| RT Shutdown Cost Multiplier | RT_SHUTDOWN_COST_MULTIPLIER | 2376.12 | NUMBER |
| Daily Load Response Emergency Credits ($) | DAILY_LR_EMERGENCY_CREDIT | 2245.12 | NUMBER(22,2) |
| Emergency Load Response Make-Whole Credits ($) | EMERGENCY_LR_MKWH_CREDIT | 2245.02 | NUMBER(22,2) |
| Version | VERSION | 4000.07 | VARCHAR2(12) |

# CSV Report Example

See Excel file titled “Emergency Load Response Make-Whole Credits.csv”

# XML Report Example

See XML file titled “Load Response Make-Whole Credits.xml”

# Hyperlinks

The online version of this report does not contain hyperlinks.

# Supporting Calculations

Emergency Load Response Make Whole Credit (2245.02) = Emergency Bid (2245.11) + (RT Shutdown Cost (2376.11) * RT Shutdown Cost Multiplier (2376.12)) – Daily Load Response Emergency Credits (2245.12)