MSRS Report Format Documentation

Load InSchedule With and Without Losses

Version 5

Revision History

| Date | Revision | Description |
| --- | --- | --- |
| 09/04/2007 | 1 | Initial Distribution |
| 10/1/2007 | 2 | Update to data type for DATE column |
| 10/12/2007 | 3 | '-Updated Possible Transaction Type values from WLR and RLR to Wholesale Load Responsibility and Retail Load Responsibility. -Clarified that for those hours where a given transaction has no MWh, the MWh value displayed will be 0. -Updated DST treatment for the Spring DST switch date to state that 0 values will be populated for the missing hour ending 3. Also updated treatment of second HE2 on all days except the Fall DST switch date to state that 0 values will be populated for this extra hour. |
| 11/12/2012 | 4 | With the 11/9/2012 retirement of eSchedules: The Load eSchedules With and Without Losses report is renamed Load InSchedule With and Without Losses. Report Column eSchedules ID (4000.10) is renamed InSchedule ID (4000.10) |
| 2/21/2020 | 5 | Removed all references to eSchedule(s) and/or eSchedule(s) reports. |

# Report

MSRS Report Name: Load InSchedule With and Without Losses

Report short name for User Interface: Load InSchedule With and Without Losses

Download File Name Abbreviation: LdInSched

Data Granularity: Hourly

Frequency: Updated Daily

Range Displayed on Report: Start Date through End Date

# Supported Billing Line Items

- This report does not specifically support any Billing Line Items.

# Report Content Summary

This report displays the load MWh with and without losses for each load Inschedule (WLRs and RLRs) applicable to the customer account.

# Summary of Changes and Special Logic

- The hours will not be labeled in GMT in order to avoid the misalignment of EPT and GMT hours and the subsequent need to repeat the column headers in order to realign the column labels once the DST switch occurs. All hours listed will be Hour Ending in the EPT time zone.

- Zeros will be populated for all hours where the transaction displayed does not have any MWh for the given hour.

- Daylight Saving Time (DST) Handling

- For online versions:

  - The online version of the report will display the number of hours in each day.  24 hours will appear for all days of the year except for DST switch days.  On the Spring DST switch day, only 23 hours will appear on the report (EPT hour ending 3 is skipped); on the Fall DST switch day, 25 hours will appear on the report (EPT hour ending 02 appears twice – the second hour ending 02 will be labeled 02* and come after the first hour ending 02).

- For download versions:

  - A 25th hour column (a second EPT Hour Ending 2) will appear in the report for all days, regardless of whether it is a DST switch day or not

    - This additional column will be placed immediately after the first hour ending 2

    - It will be name ‘EPT HE 02*’ for the CSV downloads and ‘EPT_HE_02X’ for the XML downloads

  - All non-DST switch days will display ‘0’ values in the second EPT HE 2 column

  - The Fall DST switch will be handled by populating the second HE 2 column

  - The Spring DST switch will be handled by displaying ‘0’ values in the second HE 2 and the HE 3 columns

# Report Columns

The following columns will appear in the body of the report:

| Online and CSV Column Name | XML Column Name | Column Number | Data Type |
| --- | --- | --- | --- |
| Customer ID | CUSTOMER_ID | 4000.01 | INTEGER |
| Customer Code | CUSTOMER_CODE | 4000.02 | VARCHAR2(6) |
| Date | DATE | 4000.04 | DATE (MM/DD/YYYY in online and CSV formats, YYYY-MM-DD in XML format) |
| InSchedule ID | INSCHEDULE_ID | 4000.10 | NUMBER |
| Transaction Type | TRANSACTION_TYPE | 4000.11 | VARCHAR2(24) See possible values below |
| Status | STATUS | 4000.16 | VARCHAR2(19) See possible values below |
| Seller | SELLER | 4000.18 | VARCHAR2(6) |
| Buyer | BUYER | 4000.17 | VARCHAR2(6) |
| EPT HE 01 – EPT HE 24 - HE 02* | EPT_HE_01 EPT_HE_02 EPT_HE_02X EPT_HE_03 … EPT_HE_24 | 3010.01 - 3010.25 (HE 02* = 3010.25) | NUMBER |
| Total MWh | TOTAL_MWH | 3010.26 | NUMBER |
| Version | VERSION | 4000.07 | VARCHAR2(12) |

Possible Transaction Type values:  Wholesale Load Responsibility, Retail Load Responsibility

Possible Status values: Load with Losses, Load without Losses

# CSV Report Example

See Excel file titled “Load InSchedule With and Without Losses CSV Format.csv”

# XML Report Example

See XML file titled “Load InSchedule With and Without Losses XML Format.xml”

# Hyperlinks

This report contains no hyperlinks.

# Supporting Calculations

This report does not have any calculated values that are supported purely by other columns on this report.