MSRS Report Format Documentation

Load Response Summary (Pre FERC 745 (04.01.2012))

Version 4

Revision History

| Date | Revision | Description |
| --- | --- | --- |
| 03/06/2012 | 1 | Initial Distribution |
| 03/16/2012 | 2 | Updated Supported calculations to comply with FERC 745 compliance filing on 3/14/2012. |
| 3/26/2014 | 3 | Updated to remove Emergency Load Response Credit from Report View. Emergency Load Response Credit is included on the Emergency Load Response Credit report as of 3/1/2014. |
| 8/29/2014 | 4 | Updated Report Name and Report Short Name to better reflect how report is displayed in the MSRS user interface. |

# Report

MSRS Report Name: Load Response Summary (Pre FERC 745 (04.01.2012))

Related eSchedules Report Name (old):  Load Response Monthly Summary Report

Report short name for User Interface: Load Response Summary (Pre FERC 745 (04.01.2012))

Download File Name Abbreviation: LRChCr

Data Granularity: Hourly

Frequency: Updated daily

Range Displayed on Report: Start Month through End Month

# Supported Billing Line Items

- Day-ahead Economic Load Response Charge (1240)

- Day-ahead Economic Load Response Credit (2240)

- Real-time Economic Load Response Charge (1241)

- Real-time Economic Load Response Credit (2241)

# Report Content Summary

This report will display details of the energy Load Response market changes and credits for activity after 4/1/2012.  This report displays a customer account’s hourly CSP load response credits and directly allocated LSE load response charges for each end use customer for all hours processed within the selected month where the Day-Ahead Load Response Credit, DA Load Response Charge, RT Load Response Credit, and RT Load Response Charge is not zero for the hour.  This report displays a customer account’s daily operating reserves for load response charges by end use customer for all reductions which occurred after 4/1/2012.

# Summary of Changes and Special Logic

- The date range total row will only appear in the online version of the report.  It will not appear in the CSV and XML versions of the report.

- The Report will now display EPT and GMT Hour Ending to replace the eSchedules report’s day, hour-ending, and DST columns.

- The report column names will only appear once at the top of the report.  They will not be repeated at the beginning of the data for each day as they are in the eSchedules version of this report.

- If any of the following columns are not applicable to the resource listed in a given row, 0s will be displayed in these columns: DA Load Response MWh, DA Retail Rate Used, DA Load Response Credit, DA Load Response Charge, CBL, Metered Load, Load Response Loss Factor, RT Load Response MWh, RT Retail Rate Used, RT Load Response Credit, RT Load Response Charge

- The data on this report is preliminary until the end of the billing cycle.  Final results are based on month-end calculations that could significantly change the preliminary results that are displayed throughout the month.

# Report Columns

The following columns will appear in the body of the report:

| Online and CSV Column Name | XML Column Name | Column Number | Data Type |
| --- | --- | --- | --- |
| Customer ID | CUSTOMER_ID | 4000.01 | INTEGER |
| Customer Code | CUSTOMER_CODE | 4000.02 | VARCHAR2(6) |
| Billing Month | BILLING_MONTH | 4000.03 | DATE (Month, YYYY (ex. January, 2007) in online and CSV formats, YYYY-MM in XML format) |
| EPT Hour Ending | EPT_HOUR_ENDING | 4000.05 | VARCHAR2(40) mm/dd/yyyy HH24 format (Displays first hour of the day as hour 1 and last hour of the day as hour 24) |
| GMT Hour Ending | GMT_HOUR_ENDING | 4000.06 | VARCHAR2(40) mm/dd/yyyy HH24 format (Displays first hour of the day as hour 1 and last hour of the day as hour 00 of the following day) |
| Registration ID | REGISTRATION_ID | 4000.93 | NUMBER |
| EDC Account Number | EDC_ACCOUNT_NUMBER | 4000.94 | VARCHAR2(25) |
| End Use Customer | END_USE_CUSTOMER | 4000.95 | VARCHAR2(40) |
| Zone | ZONE | 4000.32 | VARCHAR2(50) |
| DA Load Response MWh | DA_DSR_MWH | 3001.14 | NUMBER(11,3) |
| DA LMP ($/MWh) | DA_LMP | 3000.24 | NUMBER(12,6) |
| DA Retail Rate Used ($/MWh) | DA_DSR_RETAIL_RATE_USED | 1240.11 | NUMBER |
| DA Load Response Credit ($) | DA_DSR_CREDIT | 2240.01 | NUMBER(22,2) |
| DA Load Response Charge ($) | DA_DSR_CHARGE | 1240.01 | NUMBER(22,2) |
| CBL (kWh) | CBL | 1241.11 | NUMBER |
| Metered Load (kWh) | METERED_LOAD | 1241.12 | NUMBER |
| Load Response Loss Factor | LOAD_RESPONSE_LOSS_FACTOR | 1241.13 | NUMBER(6,5) |
| EDC Loss De-ration Factor | EDC_LOSS_DE_RATION_FACTOR | 3000.83 | NUMBER(21,9) |
| RT Load Response MWh | RT_DSR_MWH | 3001.15 | NUMBER(22,3) |
| RT LMP ($/MWh) | RT_LMP | 3000.25 | NUMBER(12,6) |
| RT Retail Rate Used ($/MWh) | RT_DSR_RETAIL_RATE_USED | 1241.14 | NUMBER |
| RT Load Response Credit ($) | RT_DSR_CREDIT | 2241.01 | NUMBER(22,2) |
| RT Load Response Charge ($) | RT_DSR_CHARGE | 1241.01 | NUMBER(22,2) |
| Version | VERSION | 4000.07 | VARCHAR2(12) |

# CSV Report Example

See Excel file titled “Load Response Summary CSV Format.csv”

# XML Report Example

See XML file titled “Load Response Summary XML Format.xml”

# Hyperlinks

The online version of this report does not contain hyperlinks.

# Supporting Calculations

If DA LMP >= Net Benefit Price (found on PJM.com)

DA Retail Rate Used = 0

Else

DA Retail Rate Used = G&T rate

If DA LMP >= Net Benefit Price (found on PJM.com)

0

Else

DA Load Response Charge (1240.01) = DA Load Response MWh (3001.14) * MAX (DA LMP (3000.24) - DA Retail Rate Used (1240.11), 0)

DA Load Response Credit (2240.01) = DA Load Response MWh (3001.14) * MAX(DA LMP (3000.24) - DA Retail Rate Used (1240.11), 0)

RT Load Response MWh (3001.15) = Load Response Loss Factor (1241.13) * (1 – EDC Loss De-ration Factor (3000.83)) * ((CBL (1241.11) - Metered Load (1241.12)) / 1000)

If DA Load Response MWh >0

RT Retail Rate Used = 0

RT Load Response Credit (2241.01) = (RT Load Response MWh (3001.15) - DA Load Response MWh (3001.14)) * RT LMP (3000.25)

RT Load Response Charge (1241.01) = (RT Load Response MWh (3001.15) - DA Load Response MWh (3001.14)) * RT LMP (3000.25)

If DA Load Response MWh = 0

If RT LMP >= Net Benefit Price

RT Retail Rate Used = 0

RT Load Response Charge = 0

RT Load Response Credit (2241.01) = (RT Load Response MWh (3001.15) - DA Load Response MWh (3001.14)) * RT LMP (3000.25)

If RT LMP < Net Benefit Price

RT Retail Rate Used = G&T rate

RT Load Response Charge (1241.01) = (RT Load Response MWh (3001.15) - DA Load Response MWh (3001.14)) * MAX(RT LMP (3000.25) - RT Retail Rate Used (1240.11), 0)

RT Load Response Credit (2241.01) = (RT Load Response MWh (3001.15) - DA Load Response MWh (3001.14)) * MAX (RT LMP (3000.25) - RT Retail Rate Used (1240.11), 0)