MSRS Report Format Documentation

Preliminary Real-Time Daily External Energy Transactions

Version 4

Revision History

| Date | Revision | Description |
| --- | --- | --- |
| 09/04/2007 | 1 | Initial Distribution |
| 10/1/2007 | 2 | Update to data type for DATE column |
| 10/12/2007 | 3 | '- Deleted the following sentences from the section that discusses the treatment of Wheel Ins and Wheel Outs as the columns mentioned do not exist on this report: "The transmission reservation and source/sink information associated with the transaction will populate on the Wheel In transaction only. The Wheel Out transaction will have a null OASIS ID, Source PNODE Name and ID and Sink PNODE Name and ID." -Updated DST treatment for the Spring DST switch date to state that 0 values will be populated for the missing hour ending 3. Also updated treatment of second HE2 on all days except the Fall DST switch date to state that 0 values will be populated for this extra hour. - Clarified that for those hours where a given transaction has no MWh, the MWh value displayed will be 0. |
| 2/21/2020 | 4 | Removed all references to eSchedule(s) and/or eSchedule(s) reports. |

# Report

MSRS Report Name: Preliminary Real-Time Daily External Energy Transactions

Report short name for User Interface: Prelim RT Transactions

Download File Name Abbreviation: PreRTTrans

Data Granularity: Hourly

Frequency: Updated Daily

Range Displayed on Report: Start Date through End Date

# Supported Billing Line Items

This report does not specifically support any Billing Line Items.

# Report Content Summary

This report displays the customer account’s external real-time energy transactions by hour for use in their transaction verification process.

# Summary of Changes and Special Logic

- This report will be available by 8 AM EPT the day following the transaction date.

- The data on this report is preliminary and provided only for the purpose of verifying transactions.  Final results will be available on the Real-Time Daily Energy Transactions report.  The Real-Time Daily Energy Transactions report should be used for all final settlement purposes.

- Zeros will be populated for all hours where the transaction displayed does not have any MWh for the given hour.

- Wheels will be represented by two transactions: a Wheel In segment and a Wheel Out segment.

- An asterisk (*) no longer appears in the buyer/seller fields as part of the customer account’s short name when the transaction affects the account’s interchange.

- Daylight Savings Time (DST) Handling

- For online versions:

  - The online version of the report will display the number of hours in each day.  24 hours will appear for all days of the year except for DST switch days.  On the Spring DST switch day, only 23 hours will appear on the report (EPT hour ending 3 is skipped); on the Fall DST switch day, 25 hours will appear on the report (EPT hour ending 02 appears twice – the second hour ending 02 will be labeled 02* and come after the first hour ending 02).

- For download versions:

  - A 25th hour column (a second EPT Hour Ending 2) will appear in the report for all days, regardless of whether it is a DST switch day or not

    - This additional column will be placed immediately after the first hour ending 2

    - It will be name ‘EPT HE 02*’ for the CSV downloads and ‘EPT_HE_02X’ for the XML downloads

  - All non-DST switch days will display ‘0’ values in the second EPT HE 2 column

  - The Fall DST switch will be handled by populating the second HE 2 column

  - The Spring DST switch will be handled by displaying ‘0’ values in the second HE 2 and the HE 3 columns

# Report Columns

The following columns will appear in the body of the report:

| Online and CSV Column Name | XML Column Name | Column Number | Data Type |
| --- | --- | --- | --- |
| Customer ID | CUSTOMER_ID | 4000.01 | INTEGER |
| Customer Code | CUSTOMER_CODE | 4000.02 | VARCHAR2(6) |
| Date | DATE | 4000.04 | DATE (MM/DD/YYYY in online and CSV formats, YYYY-MM-DD in XML format) |
| Transaction Type | TRANSACTION_TYPE | 4000.11 | VARCHAR2(9) See possible values below |
| Transaction ID | TRANSACTION_ID | 4000.09 | VARCHAR2(40) |
| NERC Tag | NERC_TAG | 4000.13 | VARCHAR2(40) |
| Interface | INTERFACE | 4000.31 | VARCHAR2(6) |
| EPT HE 01 – EPT HE 24 - HE 02* | EPT_HE_01 EPT_HE_02 EPT_HE_02X … EPT_HE_24 | 3009.01 - 3009.25 (HE 02* - 3009.26) | NUMBER |
| Total MWh | TOTAL_MWH | 3009.26 | NUMBER |

Possible Transaction Type values:  Import, Export, Wheel In, Wheel Out

# CSV Report Example

See Excel file titled “Preliminary Real-Time Daily External Energy Transactions CSV Format.csv”

# XML Report Example

See XML file titled “Preliminary Real-Time Daily External Energy Transactions XML Format.xml”

# Hyperlinks

This report contains no hyperlinks.

# Supporting Calculations

This report does not have any calculated values that are supported purely by other columns on this report.