MSRS Report Format Documentation

Schedule 9 and 10 Charge Details

Version 7

Revision History

| Date | Revision | Description |
| --- | --- | --- |
| 09/04/2007 | 1 | Initial Distribution |
| 10/12/2007 | 2 | Updated DST treatment for the Spring DST switch date to state that 0 values will be populated for the missing hour ending 3. Also updated treatment of second HE2 on all days except the Fall DST switch date to state that 0 values will be populated for this extra hour. |
| 1/14/2008 | 3 | Updated the XML column names for the EPT HE 1 – EPT HE 24 columns. The Sched_9_ prefix was removed from the column names in this document to match the sample XML file that was published. |
| 11/12/2008 | 4 | Updated to include 2 rows for Schedule 9-MMU: Market Monitoring Unit (MMU) Funding. |
| 1/2/2018 | 5 | Updated to include rows for Consumer Advocates of PJM States, Inc. (CAPS) |
| 11/26/2019 | 6 | Updated Schedule and Determinant values to include Non-Dispatch Charging Energy (MWh) |
| 2/21/2020 | 7 | Removed all references to eSchedule(s) and/or eSchedule(s) reports |

# Report

MSRS Report Name: Schedule 9 and 10 Charge Details

Report short name for User Interface: Schedule 9 and 10 Charge Details

Download File Name Abbreviation: 9&10ChDet

Data Granularity: Daily

Frequency: Updated Daily

Range Displayed on Report: Start Date through End Date

# Supported Billing Line Items

- PJM Scheduling, System Control and Dispatch Service - Control Area Administration Charge (1301)

- PJM Scheduling, System Control and Dispatch Service - FTR Administration Charge (1302)

- PJM Scheduling, System Control and Dispatch Service - Market Support Charge (1303)

- PJM Scheduling, System Control and Dispatch Service - Regulation Market Administration Charge (1304)

- PJM Scheduling, System Control and Dispatch Service - Capacity Resource/Obligation Mgmt. Charge (1305)

- PJM Scheduling, System Control and Dispatch Service Refund - Control Area Administration Charge (1308)

- PJM Scheduling, System Control and Dispatch Service Refund - FTR Administration Charge (1309)

- PJM Scheduling, System Control and Dispatch Service Refund - Market Support Charge (1310)

- PJM Scheduling, System Control and Dispatch Service Refund - Regulation Market Administration Charge (1311)

- PJM Scheduling, System Control and Dispatch Service Refund - Capacity Resource/Obligation Mgmt. Charge (1312)

- Market Monitor Unit (MMU) Funding (1314)

- FERC Annual Charge Recovery Charge (1315)

- Organization of PJM States, Inc. (OPSI) Funding Charge (1316)

- North American Electric Reliability Corporation (NERC) Charge (1317)

- Reliability First Corporation (RFC) Charge (1318)

- Consumer Advocates of PJM States, Inc. (CAPS) (1319)

# Report Content Summary

This report displays the details behind the customer account’s daily Schedule 9-1 through 9-5, 9-MMU, 9-FERC, 9-OPSI, 9-CAPS, 10-NERC and 10-RFC Charges.  The details behind the account’s Schedule 9 Refund Charges are also displayed.

# Summary of Changes and Special Logic

- The report will display daily values.

- The hours on this report will not be labeled in GMT in order to avoid the misalignment of EPT and GMT hours and the subsequent need to repeat the column headers in order to realign the column labels once the DST switch occurs. All hours listed will be Hour Ending in the EPT time zone.

- Refund Rates will appear in an additional column beside the original rate column rather than being combined with the original rate.

- Daily schedule 9 & 10 charges will not appear on this report.  This report will show all details behind the charges.  Daily charges can be retrieved from the Month-to-Date Billing Statement.

- Daylight Saving Time (DST) Handling

- For online versions:

  - The online version of the report will display the number of hours in each day.  24 hours will appear for all days of the year except for DST switch days.  On the Spring DST switch day, only 23 hours will appear on the report (EPT hour ending 3 is skipped); on the Fall DST switch day, 25 hours will appear on the report (EPT hour ending 02 appears twice – the second hour ending 02 will be labeled 02* and come after the first hour ending 02).

- For download versions:

  - A 25th hour column (a second EPT Hour Ending 2) will appear in the report for all days, regardless of whether it is a DST switch day or not

    - This additional column will be placed immediately after the first hour ending 2

    - It will be name ‘EPT HE 02*’ for the CSV downloads and ‘EPT_HE_02X’ for the XML downloads

  - All non-DST switch days will display ‘0’ values in the second EPT HE 2 column for those determinants that have hourly use values

  - The Fall DST switch will be handled by populating the second HE 2 column for those determinants that have hourly use values

  - The Spring DST switch will be handled by displaying ‘0’ values in the second HE 2 and the HE 3 columns for those determinants that have hourly use values

# Report Columns

The following columns will appear in the body of the report:

| Online and CSV Column Name | XML Column Name | Column Number | Data Type |
| --- | --- | --- | --- |
| Customer ID | CUSTOMER_ID | 4000.01 | INTEGER |
| Customer Code | CUSTOMER_CODE | 4000.02 | VARCHAR2(6) |
| Date | DATE | 4000.04 | DATE (MM/DD/YYYY in online and CSV formats, YYYY-MM-DD in XML format) |
| Determinant | DETERMINANT | 4000.75 | VARCHAR2(75) (See Predetermined values below) |
| Schedule | SCHEDULE | 4000.74 | VARCHAR2(75) (See Predetermined values below) |
| Rate | RATE | 3012.27 | NUMBER |
| Refund Rate | REFUND_RATE | 3012.28 | NUMBER |
| Daily Use | DAILY_USE | 3012.26 | NUMBER |
| EPT HE 01 – EPT HE 24 - HE 02* | EPT_HE01 EPT_HE02 EPT_HE02X … EPT_HE24 | 3012.01 - 3012.25 (HE 02* - 3012.25) | NUMBER |
| Version | VERSION | 4000.07 | VARCHAR2(12) |

Predetermined Schedule and Determinate values (all of these rows will appear for each day, regardless of whether the customer account has a Daily Use value for that schedule):

| Schedule | Determinant |
| --- | --- |
| 9-1: Control Area Administration | RT Load with Losses (MWh) |
| 9-1: Control Area Administration | RT PTP Transmission Use (MWh) |
| 9-2: Financial Transmission Rights | Financial Transmission Rights (MW) |
| 9-2: Financial Transmission Rights | FTR Bid Obligations |
| 9-2: Financial Transmission Rights | FTR Bid Options X5 |
| 9-3: Market Support | DA Cleared Increment Offers (MWh) |
| 9-3: Market Support | DA Cleared Decrement Bids (MWh) |
| 9-3: Market Support | DA Cleared Up To Congestion Energy (MWh) |
| 9-3: Market Support | RT Generation (MWh) |
| 9-3: Market Support | RT Load with Losses (MWh) |
| 9-3: Market Support | RT Network Import Use (MWh) |
| 9-3: Market Support | RT Spot Import Use (MWh) |
| 9-3: Market Support | RT PTP Transmission Use Excluding Wheel-Throughs (MWh) |
| 9-3: Market Support | Generation Offer Segments |
| 9-3: Market Support | DA Market Bid/Offer Segments |
| 9-3: Market Support | Non-Dispatch Charging Energy (MWh) |
| 9-4: Regulation and Frequency Response | Regulation Assigned (MWh) |
| 9-4: Regulation and Frequency Response | Regulation Obligation (MWh) |
| 9-5: Capacity Resource and Obligation Management | Unforced Capacity including FRR (MW) |
| 9-5: Capacity Resource and Obligation Management | UCAP Obligation (MW) |
| 9-MMU: Market Monitoring Unit (MMU) Funding | 9-3 Energy (MWh) |
| 9-MMU: Market Monitoring Unit (MMU) Funding | 9-3 Bid/Offer Segments |
| 9-FERC: FERC Annual Charge Recovery | RT Load with Losses (MWh) |
| 9-FERC: FERC Annual Charge Recovery | RT PTP Transmission Use (MWh) |
| 9-FERC: FERC Annual Charge Recovery | Non-Dispatch Charging Energy (MWh) |
| 9-OPSI: Organization of PJM States, Inc Funding | RT Load with Losses (MWh) |
| 9-OPSI: Organization of PJM States, Inc Funding | RT PTP Transmission Use (MWh) |
| 9-OPSI: Organization of PJM States, Inc Funding | Non-Dispatch Charging Energy (MWh) |
| 9-CAPS: Consumer Advocates of PJM States, Inc. | RT Load with Losses (MWh) |
| 9-CAPS: Consumer Advocates of PJM States, Inc. | Non-Dispatch Charging Energy (MWh) |
| 10-NERC: North American Electric Reliability Corporation Charge | RT RFC Load with Losses (MWh) |
| 10-NERC: North American Electric Reliability Corporation Charge | Non-Dispatch Charging Energy (MWh) |
| 10-RFC: Reliability First Corporation Charge | RT RFC Load with Losses (MWh) |
| 10-RFC: Reliability First Corporation Charge | Non-Dispatch Charging Energy (MWh) |

# CSV Report Example

See Excel file titled “Schedule 9 and 10 Charge Details CSV Format.csv”

# XML Report Example

See XML file titled “Schedule 9 and 10 Charge Details XML Format.xml”

# Hyperlinks

This report contains the following hyperlinks:

| Schedule Name | Column Name | Hyperlinked Report |
| --- | --- | --- |
| 9-1 Control Area Administration | Daily Use | Real-Time Transmission Service Usage |
| 9-3 Market Support | Daily Use | Day-Ahead Daily Energy Transactions |
| 9-3 Market Support | Daily Use | Day-Ahead Daily Energy Transactions |
| 9-3 Market Support | Daily Use | Day-Ahead Daily Energy Transactions |
| 9-3 Market Support | Daily Use | Real-Time Transmission Service Usage |
| 9-3 Market Support | Daily Use | Real-Time Transmission Service Usage |
| 9-3 Market Support | Daily Use | Real-Time Transmission Service Usage |
| 9-4: Regulation and Frequency Response | Daily Use | Regulation Summary |
| 9-4: Regulation and Frequency Response | Daily Use | Regulation Summary |
| 9-5: Capacity Resource and Obligation Management | Daily Use | Reliability Charge Summary |
| 9-FERC: FERC Annual Charge Recovery | Daily Use | Real-Time Transmission Service Usage |
| 9-OPSI: | Daily Use | Real-Time Transmission Service Usage |

# Supporting Calculations

This report does not have any calculated values that are supported purely by other columns on this report.